# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

### COURSE NAME: Brasilla R	COMPUTER ARCHITECTURE COURSE CODE:	CSA12

### COURSE OUTCOME COVERED

CO1: Components of a computer system, including CPU, memory, and I/O. Basic Operational Concepts: Instruction execution cycle, instruction fetch and decode.Bus Structures: Single-bus, multi-bus, and hierarchical buses.Factors of Performance: CPU execution time, CPI, clock cycles, and benchmarking.8085 and 8086 Architectures: Overview, instruction sets, and addressing modes. Modern Architectures: ARM Cortex, Intel Core, and AMD Ryzen.Instruction Sets, Formats, and Addressing Modes. Number Systems: Binary, hexadecimal, and their applications in computer systems. (BL4)

Assessment Tool Weightage: 40%

### Total Marks:25

ANNEXURE A APPLICATION BASED QUESTIONS

Code of Conduct:

I Brasilla R (192511357)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Brasilla R

### ANNEXURE A

- A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

- A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch–decode–execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

- An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

- A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

- A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

### MARKS ALLOCATION

| Criteria | Understanding of Problem Context (20%) | Application of Concepts (20%) | Problem-Solving Approach (20%) | Accuracy of Solution (20%) | Clarity (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

### RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understan ding of Problem Context | 20 | Clearly understands the problem and accurately identifies all requirements | Good understanding with minor gaps | Basic understanding; some requirements unclear | Poor understanding or misinterpretation of the problem |
| Application of Concepts | 30 | Accurately applies appropriate concepts to solve complex problems | Applies relevant concepts correctly with minor errors | Applies basic concepts with limited accuracy | Fails to apply appropriate concepts |
| Problem-Solving Approach | 20 | Logical, well-structured, and efficient approach | Mostly logical approach with minor gaps | Basic approach with some inconsistencies | Illogical or unclear approach |
| Accuracy of Solution | 20 | Completely correct solution with proper justification | Mostly correct with minor errors | Partially correct solution | Incorrect or incomplete solution |
| Clarity | 10 | Clear, well-organized, and properly explained solution | Understandable with minor clarity issues | Limited clarity and explanation | Poorly presented and difficult to understand |

1.A smart city surveillance system processes continuous video feeds from multiple cameras, where large volumes of data are transferred between I/O devices, memory, and CPU. During peak hours, the system experiences lag and delayed response in threat detection. Analyze how the interaction between CPU, memory, and I/O contributes to this issue, and explain how different bus structures (single-bus vs multi-bus) and improvements in instruction execution cycle can enhance system performance.

Introduction

A smart city surveillance system continuously receives video data from multiple cameras. The CPU processes the data, memory stores frames temporarily, and I/O devices transfer video streams. During peak hours, the huge amount of data creates bottlenecks, causing delays in threat detection.

## 1. Interaction Between CPU, Memory, and I/O

### CPU (Central Processing Unit)

- Processes video frames.

- Executes image processing and AI algorithms.

- Makes threat detection decisions.

### Memory

- Temporarily stores video frames.

- Provides data quickly to the CPU.

- Large memory reduces delays.

### I/O Devices

- Cameras continuously send video data.

- Network interfaces receive and transmit data.

- Storage devices save recorded videos.

### Working Process

- Cameras capture video.

- Video data is transferred through I/O.

- Memory stores the incoming frames.

- CPU fetches frames from memory.

- CPU processes and detects threats.

## 2. Causes of Lag During Peak Hours

- Large number of cameras generating continuous data.

- CPU becomes overloaded while processing many video streams.

- Memory bandwidth becomes insufficient.

- Bus congestion delays data transfer.

- Frequent memory access increases waiting time.

- Slow instruction execution increases response time.

## 3. Role of Bus Structure

### 3. Role of Bus Structure

### Single-Bus Structure

Definition:
 One common bus is shared by CPU, memory, and I/O devices.

Advantages

- Simple design.

- Low cost.

- Easy to implement.

Disadvantages

- Only one transfer at a time.

- Bus becomes a bottleneck.

- High waiting time.

- Poor performance for large data transfer.

Effect
 The surveillance system experiences delays because multiple cameras compete for the same bus.

### Multi-Bus Structure

Definition:
 Separate buses are used for CPU, memory, and I/O communication.

Advantages

- Parallel data transfer.

- Faster communication.

- Reduced bus congestion.

Effect
 Multiple video streams are processed simultaneously, reducing lag and improving threat detection.

| Feature | Single Bus Architecture | Multi-Bus Architecture |
| --- | --- | --- |
| Number of Buses | One shared bus | Multiple dedicated buses |
| Data Transfer | One at a time (sequential) | Parallel transfers |
| Speed | Low | High |
| Cost | Low | Higher |
| Performance | Lower | Better |
| Bus Congestion | High | Low |

## 4. Instruction Execution Cycle

The CPU executes every instruction in four stages.

### Step 1: Fetch

Instruction is fetched from memory.

### Step 2: Decode

Control Unit decodes the instruction.

### Step 3: Execute

ALU performs the required operation.

### Step 4: Store (Write Back)

Result is stored in memory or registers.

During heavy video processing, millions of instructions are executed repeatedly. Slow execution increases system delay.

## 5. Methods to Improve Instruction Execution

### Instruction Pipelining

- Executes multiple instructions simultaneously.

- Increases throughput.

- Reduces execution time.

### Cache Memory

- Frequently used data is stored in cache.

- Reduces memory access time.

- Speeds up CPU processing.

### Direct Memory Access (DMA)

- Transfers data between I/O and memory without CPU involvement.

- Reduces CPU workload.

- Improves efficiency.

### Multi-Core Processor

- Multiple cores process different camera feeds simultaneously.

- Improves parallel processing.

### High-Speed Memory

- DDR5 or high-speed RAM reduces memory delay.

## 6. Overall Performance Improvements

- Use a multi-bus architecture.

- Increase RAM capacity and speed.

- Use cache memory.

- Implement DMA for faster data transfer.

- Use pipelining and parallel execution.

- Upgrade to multi-core processors.

- Optimize software algorithms.

Conclusion

The lag occurs because the CPU, memory, and I/O devices compete for data transfer and processing resources. A single-bus architecture creates bottlenecks, while a multi-bus architecture allows parallel communication. Techniques such as pipelining, cache memory, DMA, and multi-core processors significantly improve instruction execution speed and overall surveillance system performance. This ensures faster and more reliable real-time threat detection.

2.A real-time stock trading application running on a processor executes millions of instructions per second, but users report delays in transaction processing during high load. The system has a high CPI due to frequent memory access and branch instructions. Examine how the fetch–decode–execute cycle impacts execution time in this scenario, and propose methods to reduce CPI and improve overall CPU performance using suitable architectural enhancements.

## Introduction

A real-time stock trading application processes millions of buy and sell transactions every second. The processor executes a large number of instructions, but during high load, frequent memory access and branch instructions increase the Cycles Per Instruction (CPI), causing delays in transaction processing.

## 1. Fetch–Decode–Execute Cycle

The CPU executes every instruction in three main stages.

### Step 1: Fetch

- The CPU fetches the instruction from memory.

- The Program Counter (PC) points to the next instruction.

- Slow memory access increases fetch time.

### Step 2: Decode

- The Control Unit decodes the instruction.

- It determines the required operation and operands.

### Step 3: Execute

- The ALU performs arithmetic or logical operations.

- Branch instructions may change the execution path.

- Results are written back to registers or memory.

## 2. Impact on Execution Time

During high trading activity:

- Millions of instructions are fetched and executed.

- Frequent memory access delays instruction fetching.

- Branch instructions interrupt the instruction flow.

## 3. Causes of High CPI

### Frequent Memory Access

- CPU spends more cycles waiting for data from RAM.

- Memory latency increases CPI.

### Branch Instructions

- Conditional branches disturb the instruction pipeline.

- Incorrect branch prediction causes pipeline flushing.

### Cache Misses

- Required data is not found in cache.

- CPU accesses slower main memory.

### Pipeline Stalls

- Data hazards and control hazards stop instruction execution temporarily.

## 4. Methods to Reduce CPI

### Instruction Pipelining

- Executes multiple instructions simultaneously.

- Improves throughput.

- Reduces average CPI.

### Cache Memory

- Stores frequently used instructions and data.

- Reduces memory access time.

- Lowers CPI.

### Branch Prediction

- Predicts the outcome of branch instructions.

- Reduces pipeline stalls.

- Improves execution speed.

### Out-of-Order Execution

- CPU executes independent instructions while waiting for others.

- Increases CPU utilization.

### Superscalar Architecture

- Executes multiple instructions in one clock cycle.

- Improves performance.

### Prefetching

- Fetches future instructions before they are needed.

- Reduces waiting time.

5. Architectural Enhancements

- Increase cache size (L1/L2/L3) to reduce memory access.

- Use multi-core processors to execute multiple trading tasks in parallel.

- Use high-speed RAM to reduce memory latency.

- Improve branch prediction algorithms to minimize mispredictions.

- Optimize compiler techniques for efficient instruction scheduling.

## 6. Benefits of Reducing CPI

- Faster transaction processing.

- Reduced response time.

- Higher CPU efficiency.

- Better throughput.

- Improved user experience.

- Reliable real-time trading performance.

## Conclusion

In a real-time stock trading system, high CPI caused by frequent memory access and branch instructions increases execution time and delays transactions. The fetch–decode–execute cycle plays a crucial role in CPU performance. Architectural enhancements such as instruction pipelining, cache memory, branch prediction, superscalar processors, and multi-core architecture reduce CPI and improve overall CPU performance, ensuring fast and reliable transaction processing.

3.An embedded system based on an 8085 microprocessor is used in an industrial temperature control unit, where sensors provide input and actuators respond accordingly. However, incorrect temperature readings are occasionally processed due to improper use of addressing modes in the program. Discuss how different addressing modes influence data access, identify possible causes of such errors, and suggest how the instruction set of 8085 can be effectively used to ensure accurate and reliable operation.

## Introduction

The 8085 microprocessor is widely used in embedded systems for monitoring and controlling industrial processes. In a temperature control unit, sensors provide temperature values as input, and actuators such as heaters or coolers respond based on these values. Incorrect use of addressing modes may cause wrong data access, leading to inaccurate temperature readings and improper control actions.

## 1. Working of the Temperature Control System

- Temperature sensors measure the current temperature.

- Sensor data is sent to the 8085 microprocessor.

- The CPU processes the data.

- The result is compared with the required temperature.

- Actuators (heater/cooler) are controlled accordingly.

## 2. Addressing Modes in 8085

Addressing modes determine how data is accessed by the processor.

### a) Immediate Addressing Mode

- Data is directly specified in the instruction.

Example:

MVI A, 30H

- Loads 30H into register A.

Use:
 Setting reference temperature values.

### b) Register Addressing Mode

- Data is stored in registers.

Example:

MOV A, B

- Copies data from register B to A.

Use:
 Fast processing of sensor values.

### c) Direct Addressing Mode

- Memory address is directly given.

Example:

LDA 2500H

- Loads data from memory location 2500H.

Use:
 Reading stored sensor data.

### d) Register Indirect Addressing Mode

- Memory address is stored in register pair.

Example:

MOV A, M

- Loads data from memory pointed by HL pair.

Use:
 Accessing multiple sensor readings.

### e) Implicit Addressing Mode

- Operand is implied in the instruction.

Example:

CMA

- Complements accumulator content.

## 3. Influence of Addressing Modes on Data Access

- Correct addressing provides accurate sensor values.

- Wrong addressing accesses incorrect memory locations.

- Incorrect data leads to wrong temperature calculations.

- Improper actuator control affects system performance.

4. Possible Causes of Errors

### Wrong Memory Address

- Sensor data may be read from the wrong location.

### Incorrect Register Usage

- Data may be overwritten accidentally.

### Improper HL Pair Handling

- Wrong pointer values cause invalid data access.

### Programming Errors

- Incorrect instructions lead to faulty calculations.

### Noise or Sensor Errors

- External disturbances affect readings.

## 5. Effective Use of 8085 Instruction Set

### Data Transfer Instructions

Used to move sensor data.

Examples:

MOV A, B

LDA 2500H

STA 2600H

### Arithmetic Instructions

Used for calculations.

Examples:

ADD B

SUB C

INR A

DCR A

### Logical Instructions

Used for comparison and checking.

Examples:

CMP B

ANA C

ORA D

### Branch Instructions

Used for decision-making.

Examples:

JZ COOLER_ON

JNZ HEATER_ON

JC ALERT

### Input/Output Instructions

Used to communicate with sensors and actuators.

Examples:

IN 01H

OUT 02H

## 6. Methods to Ensure Accurate Operation

- Use proper addressing modes.

- Validate sensor data before processing.

- Use interrupts for real-time operation.

- Store data correctly in memory.

- Avoid register overwriting.

- Implement error checking.

- Use calibration techniques.

## Conclusion

Addressing modes in the 8085 microprocessor play an important role in accessing sensor data correctly. Improper use can lead to inaccurate temperature readings and faulty actuator control. Effective use of data transfer, arithmetic, logical, branch, and I/O instructions ensures reliable and accurate industrial temperature control.

4.A computing system uses an 8086 microprocessor with segmented memory for executing multiple programs simultaneously, but it encounters issues like incorrect data access and stack overflow errors. Evaluate how segmentation (code, data, and stack segments) works in 8086, analyze how improper segment handling can lead to such problems, and explain how instruction execution and addressing modes must be managed to ensure correct program execution.

Introduction

The 8086 microprocessor uses a segmented memory architecture to efficiently utilize its 1 MB memory space. Memory is divided into Code Segment (CS), Data Segment (DS), Stack Segment (SS), and Extra Segment (ES). Proper management of these segments ensures correct program execution. Incorrect segment handling may lead to wrong data access, stack overflow, and program errors.

## 1. Segmentation in 8086

The 8086 divides memory into four segments:

### a) Code Segment (CS)

- Stores program instructions.

- Controlled by the CS register.

- The Instruction Pointer (IP) points to the next instruction.

Function:

- Fetches and executes program instructions.

### b) Data Segment (DS)

- Stores variables and data.

- Controlled by the DS register.

Function:

- Used for reading and writing data.

### c) Stack Segment (SS)

- Stores temporary data, return addresses, and registers.

- Controlled by the SS register.

- Uses the Stack Pointer (SP).

Function:

- Supports function calls and interrupts.

### d) Extra Segment (ES)

- Used as an additional data segment.

- Mainly used in string operations.

## 2. Working of Segmentation

The physical address is calculated using:

Physical Address = Segment Address × 10H + Offset Address

Example:

- Segment = 2000H

- Offset = 1500H

Physical Address = 2000H × 10H + 1500H = 21500H

This allows the 8086 to access 1 MB of memory.

## 3. Problems Due to Improper Segment Handling

### Incorrect Data Access

- Wrong DS value causes access to incorrect memory locations.

- Results in wrong data being processed.

### Stack Overflow

- Excessive PUSH operations without POP.

- Stack exceeds its allocated memory.

- Program may crash.

### Wrong Code Execution

- Incorrect CS or IP values cause execution of invalid instructions.

### Memory Corruption

- Overlapping segments overwrite important data.

## 4. Instruction Execution Cycle in 8086

The CPU executes instructions in the following stages:

### Step 1: Fetch

- Instruction is fetched from the Code Segment.

### Step 2: Decode

- The Control Unit decodes the instruction.

### Step 3: Execute

- The ALU performs the required operation.

### Step 4: Store

- Results are stored in registers or memory.

Proper segment values ensure that each stage accesses the correct memory location.

## 5. Addressing Modes in 8086

### Immediate Addressing

- Data is part of the instruction.

Example:

MOV AX, 1234H

### Register Addressing

- Data is stored in registers.

Example:

MOV AX, BX

### Direct Addressing

- Memory address is specified directly.

Example:

MOV AX, [2000H]

### Register Indirect Addressing

- Memory address is stored in a register.

### Based and Indexed Addressing

- Uses base and index registers to access arrays and tables.

Example:

MOV AX, [BX+SI]

## Conclusion

The 8086 segmented memory architecture allows efficient use of memory by dividing it into Code, Data, Stack, and Extra Segments. Improper segment handling can cause incorrect data access, stack overflow, and program crashes. Proper management of segment registers, instruction execution, and addressing modes ensures accurate, reliable, and efficient program execution.

5.A data communication system represents packet information in hexadecimal format for ease of debugging and transmission, but a software engineer mistakenly interprets values, leading to incorrect data processing. Analyze the importance of number system conversions between hexadecimal and binary in such systems, explain how errors in conversion can affect instruction execution, and discuss how efficient CPU design and benchmarking can help detect and minimize such issues in real-time systems.

Introduction

In data communication systems, packet information is commonly represented in hexadecimal (Hex) because it is compact and easy to read. However, the CPU processes all data in binary (0s and 1s). Therefore, correct conversion between hexadecimal and binary is essential. Errors in conversion can lead to incorrect instruction execution, data corruption, and communication failures.

## 1. Importance of Number System Conversion

### Hexadecimal Number System

- Base 16 number system.

- Uses digits 0–9 and letters A–F.

- Easier to represent large binary values.

### Binary Number System

- Base 2 number system.

- Uses only 0 and 1.

- Directly understood by the CPU.

| Hexadecimal | Binary |
| --- | --- |
| A | 1010 |
| F | 1111 |
| 2A | 0010 1010 |
| 3F | 0011 1111 |

## 2. Importance in Data Communication

- Packet headers are displayed in hexadecimal.

- CPUs process packets in binary.

- Network engineers use hexadecimal for debugging.

- Correct conversion ensures accurate packet transmission and processing.

## 3. Effects of Conversion Errors

Incorrect conversion can cause:

### Incorrect Instruction Execution

- CPU executes the wrong instruction.

### Data Corruption

- Packet contents become invalid.

### Communication Failure

- Receiver cannot interpret the packet correctly.

## 4. Effect on Instruction Execution

The CPU follows the Fetch–Decode–Execute Cycle.

### Fetch

- Instruction is fetched from memory.

### Decode

- Instruction is converted into machine language.

### Execute

- CPU performs the required operation.

If hexadecimal values are converted incorrectly into binary:

- The fetched instruction becomes invalid.

- The decoder interprets the wrong opcode.

- Execution produces incorrect results.

5. Efficient CPU Design

To minimize such errors, modern CPUs use:

### Cache Memory

- Stores frequently used data.

- Reduces memory access time.

### Instruction Pipeline

- Executes multiple instructions simultaneously.

- Improves performance.

### Error Detection Mechanisms

- Detect invalid instructions and data.

### Memory Protection

- Prevents illegal memory access.

6. Benchmarking

Benchmarking is the process of measuring CPU performance using standard tests.

### Benefits

- Detects processing errors.

- Measures execution speed.

- Compares processor performance.

- Identifies bottlenecks.

- Helps optimize system performance

Conclusion

Hexadecimal and binary number system conversions are essential in data communication systems because CPUs process data in binary while engineers use hexadecimal for debugging. Incorrect conversions can lead to wrong instruction execution, data corruption, and communication failures. Efficient CPU design, including cache memory, pipelining, error detection, and benchmarking, improves system reliability and minimizes errors in real-time applications.